Hands-On Workshop — IoT with Simulink, ThingSpeak & ESP32

Simulink IoT Monitoring Model (EN)

| ---------- | --- |
| --- | --- |
| Duration | 100 minutes (30 min IoT setup + 40 min model build + 30 min deploy & live data) |
| Audience | Undergraduate, engineering background (beginner-to-intermediate) |
| MATLAB Products | MATLAB, Simulink, Embedded Coder *(as reported by the MATLAB Dependency Analyzer)* |
| ESP32 support (add-on) | Simulink Support Package for Arduino Hardware — a support package, not a toolbox (ver will not list it) |
| Platform | ThingSpeak (MathWorks IoT analytics) over MQTT + Wi-Fi |
| Target Hardware | ESP32-WROOM (Arduino-compatible) — 1 board per group of 3 |
| Prerequisite Knowledge | Basic MATLAB (variables, scripts) and basic Simulink (blocks, simulation) |

# 1. Workshop Overview

In this 100-minute hands-on workshop you build a Simulink® model that runs on an ESP32, reads sensor data, and publishes it live to a ThingSpeak™ channel over MQTT. The model also demonstrates a closed IoT loop: it reads its own channel back and drives both a simulated and a physical output.

| Signal | Source | ThingSpeak field |
| --- | --- | --- |
| -------- | -------- | ------------------ |
| TemperatureData | Analog input (or simulation knob) | Field 1 |
| ThresholdData | Analog input / threshold knob | Field 2 |
| FanControl | TemperatureData > ThresholdData | Field 3 |

You work through the workshop in five phases:

1. ✅ Prepare the tools — verify MATLAB product set, install the ESP32 hardware support, and connect the board.

2. ✅ Create a ThingSpeak account & channel — field names, Channel ID, Write/Read/MQTT keys.

3. ✅ Explore the starter model — the simulation scaffolding is already wired (knobs, switches, displays).

4. ✅ Build the IoT + hardware blocks — analog inputs, DAC loop-back, threshold logic, MQTT publish and read-back.

5. ✅ Deploy & stream live — run in external (Monitor & Tune) mode, watch the ThingSpeak charts update.

💡 The completed model (models/SimulinkIoTThingSpeak_complete.slx) is your answer key. Use it if you fall behind — but try to build it yourself from the starter first.

# 2. Learning Objectives

After this session you will be able to:

1. ✅ Verify the required MathWorks products (MATLAB, Simulink, Embedded Coder) and the ESP32 hardware support package.

2. ✅ Create a ThingSpeak account and a private channel with named fields.

3. ✅ Locate a channel's Channel ID, Write API Key, Read API Key, and MQTT API Key.

4. ✅ Configure the ESP32 target (board selection, external mode, Wi-Fi, ThingSpeak credentials).

5. ✅ Build an IoT Simulink model: analog input, scaling, threshold logic, MQTT publish, ThingSpeak read-back.

6. ✅ Run the model in external (Monitor & Tune) mode and watch live data appear in ThingSpeak.

7. ✅ Interpret the generated embedded C code path (ERT, XCP on Serial).

# 3. Step 1 — Verify Toolboxes & Connect the ESP32 (~10 min)

Open MATLAB and run the project file first so the path and settings are configured:

>> openProject('SimulinkIoTThingSpeak.prj')

Define the model's tunable parameters before you build or run it — this creates SensorKnob and ThresholdKnob in the base workspace:

>> run('data/TunableParameter.m')   % creates SensorKnob (3072) & ThresholdKnob (2048)
>> SensorKnob.Value, ThresholdKnob.Value   % confirm they now exist

SensorKnob and ThresholdKnob are Simulink.Parameter objects with ExportedGlobal storage. The completed model and the Dashboard knobs read them from the base workspace — if you skip this, the model cannot resolve the parameters. Re-run the script whenever you reopen MATLAB.

Verify every required product is installed:

>> ver('simulink')      % Simulink
>> ver('ecoder')        % Embedded Coder

🪟 Run these in the MATLAB Command Window. A version table means the product is installed. If missing, install it from Home → Add-Ons → Get Add-Ons.

The ESP32 support is a Support Package, not a Toolbox — ver will NOT list it. Open the Add-On Explorer:

>> supportPackageInstaller

Confirm "Simulink Support Package for Arduino Hardware" is Installed (installing it enables the ESP32-WROOM board and the Wi-Fi/MQTT blocks). If not, select it and click Install (a MathWorks account is required).

Connect the ESP32 over USB and confirm the board is seen:

>> arduinolist          % lists board name + COM port

Record the COM port (e.g. COM8). The completed model uses external mode (XCP on Serial) at 921600 baud on that port.

Wiring reference — ESP32-WROOM-32 DevKit pinout. Keep this chart next to the board while you jumper the DAC loop-back and locate the FanControl LED:

Figure 6: ESP32-WROOM-32 DevKit pinout. The small white numbers along the board edge are physical header positions, not GPIO numbers — read the GPIO and ADC labels instead. This workshop uses the analogue inputs A4/A5 (GPIO32/GPIO33), the DAC loop-back DAC1/DAC2 (GPIO25/GPIO26), and the digital output D2 (GPIO2, the board's onboard LED).

# 4. Step 2 — Create a ThingSpeak Account & Channel (~10 min)

🌐 This step is done in a web browser. Use your own MathWorks / ThingSpeak account — every group creates its own channel and its own keys.

## 4a. Create the account

1. Open https://thingspeak.mathworks.com in a browser.

Figure 1: The ThingSpeak landing page. From here, click Sign In (top-right) or Get Started For Free.

2. Click Sign In. If you do not yet have a MathWorks account, click Create Account on the MathWorks sign-in page and complete the registration (name, email, password).

3. After sign-in you land on your ThingSpeak dashboard.

## 4b. Create a private channel

1. From the top navigation click Channels → My Channels.

2. Click New Channel (or + New Channel).

3. In the Channel Settings form fill in the channel fields exactly as in Table 1 — the field names must match the signals the model publishes.

| Setting | Value |
| --- | --- |
| --------- | ------- |
| Name | ESP32MQTT |
| Description | IoT workshop with ThingSpeak MQTT and ESP32 |
| Field 1 | TemperatureData |
| Field 2 | ThresholdData |
| Field 3 | FanControl |
| Field 4–8 | *(leave blank)* |

4. Click Save Channel.

Figure 2: The Channel Settings page after saving — Field 1 = TemperatureData, Field 2 = ThresholdData, Field 3 = FanControl.

## 4c. Collect the four credentials

Open the channel (Channels → My Channels → your channel), then open the API Keys tab.

Figure 3: The API Keys tab shows the Write API Key, Read API Key(s), and REST/MQTT examples. The demo channel's key values are blurred for privacy; your own channel will show its own keys.

Record these for Step 4/5 — you will enter them into the Simulink model:

| Credential | Where to find it | Demo channel value (reference only) |
| --- | --- | --- |
| ------------ | ------------------ | ------------------------------------- |
| Channel ID | Channel header | 3478187 |

⚠️ Keep keys private. The Write key lets anyone write to your channel. Use your own keys in the model — never commit another user's key.

# 5. Step 3 — Explore the Starter Model (~5 min)

>> open_system('models/SimulinkIoTThingSpeak_starter.slx')

Figure 4: The starter model opens with the simulation scaffolding already wired — dashboard knobs, a Sim/HW source selector, threshold constant, GoTo/From signal routing, and displays.

The starter already contains (read-only for this step):

- Knob / Knob1 — dashboard inputs (ScaleMax = 4095, matching the ESP32 12-bit ADC).

- SensorADC Value and ThresholdData (constants, ~3620 / ~2951 in the starter) — the simulation values the knobs tune. In the completed model these become tunable parameters SensorKnob / ThresholdKnob (defined in data/TunableParameter.m).

- Sensor Data Source — a custom tuning web component (the Sim ↔ Hardware toggle).

- Switch / Switch2 — select the Simulated input (from the knobs) or the Hardware input (from the ESP32 ADC, added in Step 5).

- GoTo/From blocks — tags SimADCTemp, SimThreshold, ADCTemp, ADCThreshold, TemperatureData, ThresholdData, FanControl carry signals between the two halves of the model.

- Displays & Lamp — visualize scaled values and on/off states.

# 6. Step 4 — Configure the ESP32 Target (~5 min)

Open Model Settings (Ctrl+E) and check the following:

| Tab | Setting | Value |
| --- | --- | --- |
| ----- | --------- | ------- |
| Solver | Type | Fixed-step |
|  | Solver | discrete (no continuous states); Fixed-step size auto |
| Hardware Implementation | Hardware board | ESP32-WROOM (Arduino Compatible) |
|  | Sample time | 0.01 (from the Analog Input blocks) |
| Hardware Implementation → Target hardware resources → External mode | Configuration | XCP on Serial |
|  | COM port | your arduinolist port (e.g. COM8) |
|  | Baud rate | 921600 |
| Hardware Implementation → Target hardware resources → Wi-Fi | SSID | your access point name |
|  | Password (WPA) | your Wi-Fi password |
| Hardware Implementation → Target hardware resources → ThingSpeak / MQTT | Channel ID | your Channel ID |
|  | Write / MQTT API key | your own keys |

Figure 5: Model Settings → Hardware Implementation (opened with Ctrl+E). The ESP32-WROOM (Arduino Compatible) board, ert.tlc code-generation target, and the target hardware resources (Wi-Fi, external mode) are configured here.

Code-generation optimization (Model Settings → Code Generation → Optimization):

- Default parameter behavior = Inlined (parameters inlined; RTWInlineParameters = on, access Literals).

- Tunable parameters — SensorKnob and ThresholdKnob are Simulink.Parameter objects with ExportedGlobal storage, defined in data/TunableParameter.m (SensorKnob.Value = 3072, ThresholdKnob.Value = 2048). Because they are tunable, you can change them live in external (Monitor & Tune) mode without a rebuild — the Dashboard knobs drive them.

- Optimization level = standard (OptimizationPriority Balanced); buffer and block-I/O storage optimization off — keeps the generated code simple and readable.

💡 The completed model ships with placeholder credentials (YOUR_WIFI_SSID, YOUR_WIFI_PASSWORD, YOUR_MQTT_USERNAME, YOUR_MQTT_PASSWORD, YOUR_READ_API_KEY). Replace them with your own Wi-Fi and channel so the data flows to your ThingSpeak account — the block dialogs and target-resource settings read these values at build time.

# 7. Steps 5–8 — Build the IoT & Hardware Blocks (~40 min)

All blocks below appear in the completed model (models/SimulinkIoTThingSpeak_complete.slx). Add them to the starter and wire them to the existing GoTo/From tags.

## Step 5: Analog inputs + DAC loop-back

From the Simulink Support Package for Arduino library (arduinolib) add:

1. Two Analog Input blocks.

- Settings: Pin = 4 and Pin = 5 (the ADC pins the sensors are wired to); Sample time = 0.01; output uint16.

Figure 7: The Analog Input dialog — Pin = 4 for the temperature sensor, Sample time = 0.01, output uint16.

2. Wire them (via the Gain 1/16 scaling and a Data Type Conversion to uint16) into the ADCTemp / ADCThreshold GoTo tags, so the Switch blocks can select them in Hardware mode.

3. Add two Analog Output blocks and set their pins to DAC1 and DAC2. Wire the scaled ADC values into them — this creates a real ADC → DAC loop-back on the ESP32: turn the potentiometer and the DAC output voltage follows it.

🔑 Why 1/16? The ESP32 ADC returns 0–4095 (12-bit). 4096 ÷ 256 = 16, so Gain = 1/16 maps the full ADC range onto the 8-bit DAC range (0–255) — no saturation, full travel.

## Step 6: Publish to ThingSpeak over MQTT

The model builds a payload string and publishes it every 60 s.

1. Add a Compose String block. In its dialog set:

- Format = field1=%d&field2=%d&field3=%d

- Inputs: wire From_TemperatureData → input 1, From_ThresholdData → input 2, From_FanControl → input 3.

Figure 8: The Compose String dialog — the Format field is the ThingSpeak payload template field1=%d&field2=%d&field3=%d.

2. The starter already has a String to ASCII block (output vector size 128). Wire the Compose String output into it — MQTT transmits the payload as an ASCII byte vector.

Figure 9: The String to ASCII dialog — Output vector size 128.

3. Add a WiFi MQTT Publish block (arduinowifilib) and set:

- Broker service = ThingSpeak

- Topic = channels/<YOUR_CHANNEL_ID>/publish

- Update interval = 60 (seconds)

- Wire the String to ASCII output into its input. Connect its Output to a Display (shows publish status / error code).

Figure 10: The WiFi MQTT Publish dialog — Broker service = ThingSpeak, Update interval = 60 s.

📡 The block uses the Wi-Fi and ThingSpeak MQTT credentials you set in Step 4 (Model Settings → Target hardware resources). ThingSpeak's MQTT broker is mqtt3.thingspeak.com, port 1883; the block keeps a persistent connection and publishes on the interval.

## Step 7: Fan / activity logic (threshold)

1. Add a Relational Operator and set Operator = >.

- Input 1: TemperatureData · Input 2: ThresholdData.

Figure 11: The Relational Operator dialog — Operator = >. Input 1 = TemperatureData, Input 2 = ThresholdData.

2. Wire its output (boolean) into the FanControl GoTo tag. The signal is already routed to:

- the Compose String (Field 3) — so the on/off state is published; and

- From_FanControl → Digital Output block on Pin 2 — drives the LED/buzzer on the board.

Figure 12: The Digital Output dialog — Pin number 2 (the board's Fan/activity output).

When TemperatureData > ThresholdData, FanControl = 1 and D2 turns on.

## Step 8: Read your own channel back (closed loop)

1. Add a WiFi ThingSpeak Read block (arduinowifilib) and set:

- Channel ID = your Channel ID

- Interface = Wi-Fi; Channel access = Private

- Read API key = your Read key

- Fields to read = [1 2 3]

- Sample time = 15 (seconds)

Figure 13: The WiFi ThingSpeak Read dialog — Channel ID, Read API key, Fields to read = [1 2 3], Sample time = 15 s.

2. Wire its Outputs 2, 3, 4 into three Display blocks. Output 1 is the request status.

This gives you a read-back loop: whatever the ESP32 publishes appears back in the Displays within ~15 s — proving the round trip through the cloud.

## Save & update

>> set_param('SimulinkIoTThingSpeak_complete','SimulationCommand','update')
>> save_system('SimulinkIoTThingSpeak_complete')

Figure 14: The completed model. Analog inputs (ADC)×2 feed the scaling/DAC loop-back and the Sim↔Hardware switches; the Compose String → String to ASCII → MQTT Publish chain exits top-centre; the Relational Operator drives FanControl; and WiFi ThingSpeak Read returns the channel's own data to the displays.

# 8. Step 9 — Deploy & Stream Live Data (~15 min)

Because external mode is enabled, MATLAB builds, uploads, and runs the model on the ESP32, then lets you Monitor & Tune over the XCP-on-Serial link.

## Step 9a — Run with Monitor & Tune (Hardware tab)

1. In the model toolstrip, open the Hardware tab.

2. In the Run on Hardware group, click Monitor & Tune.

Figure 15: On the Hardware tab, click Monitor & Tune (red circle) — MATLAB builds, uploads and starts the model on the ESP32 in external mode.

⚠️ First run: the model reads SensorKnob / ThresholdKnob from the base workspace. If the build fails with an "undefined variable" error, run run('data/TunableParameter.m') first (Step 1), then click Monitor & Tune again.

MATLAB compiles the Embedded Coder (ERT) code, flashes the ESP32 over the configured COM port, connects XCP on Serial, and the data stream starts.

While it runs you can Monitor & Tune live:

- Flip the Sensor Data Source toggle to switch between Simulated knobs and Hardware ADC inputs.

- Turn Knob / Knob1 to change SensorKnob / ThresholdKnob (TemperatureData / ThresholdData) live — both are tunable parameters (ExportedGlobal), so they update without a rebuild.

- Watch the Displays and the Lamp react in real time.

Now open ThingSpeak → Channels → My Channels → your channel → Private View:

Figure 16: Live Field 1–3 charts on the ThingSpeak channel after the ESP32 starts publishing. Field 1 = TemperatureData, Field 2 = ThresholdData, Field 3 = FanControl.

✅ Pass criterion: within ~60 s of Run, new samples appear on Field 1 and Field 2 charts, and Field 3 flips between 0/1 as you cross the threshold with the knob. The Displays fed by WiFi ThingSpeak Read refresh the same values back on the model.

# 9. Step 10 — Validate the Field Mapping

| Model signal | Block chain | ThingSpeak field | Expected on the channel |
| --- | --- | --- | --- |
| -------------- | ------------- | ------------------ | -------------------------- |
| TemperatureData | ADC/sim → scaling → Compose String input 1 | Field 1 | ~0–4095 ADC value |
| ThresholdData | ADC/sim → scaling → Compose String input 2 | Field 2 | ~0–4095 threshold |
| FanControl | TemperatureData > ThresholdData → Compose String input 3 | Field 3 | 0 or 1 |

If a field is empty or wrong:

- Field names in the channel must equal the block/variable names exactly (TemperatureData, ThresholdData, FanControl).

- The Compose String format must be field1=%d&field2=%d&field3=%d.

- The MQTT topic must be channels/<YOUR_CHANNEL_ID>/publish (your Channel ID, not the demo's 3478187).

# 10. Summary

| Phase | Command / Action | Key Takeaway |
| --- | --- | --- |
| ------- | ------------------ | -------------- |
| Verify | ver('simulink'), supportPackageInstaller, arduinolist | Know what is installed and which COM port the ESP32 uses |
| IoT setup | ThingSpeak → My Channels → New Channel | Named fields + Write/Read/MQTT keys drive the whole model |
| Configure | Hardware Implementation → ESP32-WROOM, external mode, Wi-Fi, ThingSpeak creds | One place holds every credential the blocks read at build |
| Model | Analog In / DAC, Compose String, MQTT Publish, ThingSpeak Read, > comparator | The model is just data-in → publish → read-back |
| Deploy | SimulationMode = external, Ctrl+T | External (Monitor & Tune) flashes and streams live |
| Observe | ThingSpeak Private View | The cloud confirms the complete IoT loop |

## Architecture Learned

ESP32 ADC / dashboard knob ─▶ TemperatureData, ThresholdData
              │
    TemperatureData > ThresholdData ─▶ FanControl
              │
  Compose String "field1=%d&field2=%d&field3=%d"
              │  (String to ASCII, 128)
              ▼
  WiFi MQTT Publish (ThingSpeak, channels/<ID>/publish, 30 s)
              │
              ▼
  ThingSpeak channel (Field 1,2,3)  ◀── WiFi ThingSpeak Read (15 s) ─▶ Displays

# 11. Troubleshooting

| Issue | Possible Cause | Solution |
| --- | --- | --- |
| ------- | ---------------- | ---------- |
| arduinolist empty | Board not connected / driver | Re-plug USB; check Device Manager; try another cable/port |
| Port busy at deploy | Serial Monitor or another MATLAB open | Close Serial Monitor; pick the correct COM in Model Settings |
| WiFi publish fails | Wrong SSID / password | Re-enter Wi-Fi credentials in Target hardware resources → Wi-Fi |
| No data on channel | Wrong Channel ID / keys | Verify topic channels/<ID>/publish and the Write/MQTT keys in the block + target settings |
| Field 3 always 0 | Wrong > operand wiring | Confirm input 1 = TemperatureData, input 2 = ThresholdData; flip operands to test |
| Fields populated wrongly | Field names don't match block names | Rename channel fields to exactly TemperatureData, ThresholdData, FanControl |
| Compose output empty | Format string typo / wrong inputs | Use field1=%d&field2=%d&field3=%d wired to inputs 1,2,3 in order |
| External mode won't connect | XCP settings / baud mismatch | Set Configuration = XCP on Serial, baud 921600, correct COM port |
| DAC output stuck | ADC value saturated | Ensure Gain = 1/16 and Data Type Conversion to uint16 (no saturating overflow) |

# 12. MathWorks References

- [ThingSpeak — Getting Started](https://www.mathworks.com/help/thingspeak/getting-started-with-thingspeak.html)

- [Collect Data in a New Channel](https://www.mathworks.com/help/thingspeak/collect-data-in-a-new-channel.html)

- [ThingSpeak Channels & Charts API](https://www.mathworks.com/help/thingspeak/channels-and-charts-api.html)

- [Write Data with MQTT (ESP32/ESP8266)](https://www.mathworks.com/help/thingspeak/examples.html?category=write-data)

- [Simulink Support Package for Arduino Hardware](https://www.mathworks.com/hardware-support/arduino-simulink.html)

- [MATLAB & Simulink + ThingSpeak](https://www.mathworks.com/hardware-support/thingspeak.html)

Created for Hands-On Workshop · TechSource Systems